Design and Comparative Analysis of Booth and Pipelined Wallace Tree Multipliers

1. Abstract

This report presents the design, implementation, and performance evaluation of two multiplier architectures: an 8-bit FSM-based Booth multiplier and an 8×8 pipelined Wallace Tree multiplier. The objective is to analyze trade-offs in latency, throughput, area, timing, and power using RTL simulation and FPGA synthesis results.

2. Introduction

Multipliers are critical components in digital systems such as DSP processors, ALUs, and embedded systems. Different architectures offer varying trade-offs:

- Booth Multiplier: Optimized for reduced arithmetic operations and minimal hardware usage

- Wallace Tree Multiplier: Optimized for high-speed parallel computation

This project explores both approaches and evaluates their suitability for different application domains.

3. Design Overview

3.1 Booth Multiplier (FSM-Based)

- Sequential architecture

- Uses Booth encoding (add/subtract/no-op)

- Executes over multiple clock cycles

- Controlled via ENABLE and DONE signals

3.2 Wallace Tree Multiplier (Pipelined)

- Fully parallel architecture

- Uses carry-save reduction (CSA)

- Multi-stage pipeline (6 stages)

- Continuous streaming with valid_in / valid_out

4. Methodology

The following steps were followed:

- RTL design in Verilog

- Functional simulation using testbenches

- Synthesis using FPGA toolchain

- Extraction of:

  - Timing metrics

  - Area utilization

  - Power estimation

5. Functional Verification

Booth Multiplier

- Verified using iterative test cases

- Correct outputs observed after 8 cycles

- DONE signal indicates completion

Wallace Multiplier

- Verified using continuous streaming inputs

- Queue-based validation ensures correctness

- Outputs matched expected values for all test cases

6. Performance Analysis

6.1 Latency

| Design | Latency |
| --- | --- |
| Booth | 8 cycles |
| Wallace | 6 cycles |

6.2 Throughput

| Design | Throughput |
| --- | --- |
| Booth | 1 result / 8 cycles |
| Wallace | 1 result / cycle |

7. Timing Analysis

Booth Multiplier

- Critical Path Delay ≈ 2.3 ns

- Fmax ≈ 430 MHz

- Timing constraints met

Wallace Multiplier

- Worst Negative Slack (WNS): +8.486 ns

- Clock Period: 10 ns

- Critical Path Delay ≈ 1.514 ns

- Fmax ≈ 660 MHz

Interpretation

The Wallace multiplier achieves significantly lower delay due to parallel reduction and absence of iterative control.

8. Area Utilization

| Resource | Booth | Wallace |
| --- | --- | --- |
| LUTs | 20 | 98 |
| Flip-Flops | 36 | 206 |
| DSP Blocks | 0 | 0 |

Analysis

- Booth is highly area-efficient

- Wallace incurs higher area due to:

  - Parallel datapath

  - Pipeline registers

9. Power Analysis

Wallace Multiplier (Measured)

- Total Power ≈ 0.1 W

- Static Power ≈ 0.081 W

- Dynamic Power ≈ 0.018 W

Important Note

Power estimation is based on vectorless analysis, meaning actual dynamic power may be higher under real switching conditions.

10. Comparative Evaluation

| Metric | Booth | Wallace |
| --- | --- | --- |
| Architecture | FSM (Sequential) | Pipelined (Parallel) |
| Latency | High | Low |
| Throughput | Low | High |
| Area | Low | High |
| Speed | Moderate | High |
| Complexity | Low | High |

11. Key Insights

- Wallace tree eliminates carry propagation delay during intermediate stages

- Pipelining enables continuous output without idle cycles

- Booth reduces arithmetic operations but introduces sequential delay

- Trade-off exists between area and performance

12. Applications

Booth Multiplier

- Low-power embedded systems

- Area-constrained designs

- Control-dominated architectures

Wallace Multiplier

- DSP processors

- High-speed ALUs

- Signal processing pipelines

13. Conclusion

The Wallace Tree multiplier demonstrates superior performance in terms of speed and throughput, achieving a critical path delay of approximately 1.514 ns and a throughput of one result per clock cycle. However, this comes at the cost of increased area and design complexity.

In contrast, the Booth multiplier provides a compact and resource-efficient implementation, but suffers from higher latency and reduced throughput due to its sequential nature.

Thus, the choice of multiplier architecture depends on system requirements:

- Performance-critical systems → Wallace Tree

- Area/power-constrained systems → Booth

14. References

- Digital Design and Computer Architecture – Harris & Harris

- FPGA Synthesis Reports (Vivado)

- RTL Simulation Results